3-DOF Ship Model

The autonomous surface vessel is represented by a physics-informed manoeuvring model with three degrees of freedom (3-DOF): surge, sway, and yaw. This formulation is appropriate for low-speed path-following and static-obstacle avoidance tasks, where dominant motion occurs in the horizontal plane and the effects of heave, roll, and pitch are secondary. The model combines a rigid-body formulation with calibrated hydrodynamic terms so that it remains computationally efficient for RL simulations, while still reproducing the measured speed and turning behaviour of the Bluefin vessel.

Two coordinate systems are employed. The vessel pose is expressed in an earth-fixed frame as

η=x, y, ψ⊤

where x and y denote the planar position and ψ is the heading angle. The body-fixed velocity vector is written as

ν=u, v, r⊤

where u is surge velocity, v is sway velocity, and r is yaw rate. The kinematic transformation between body-fixed motion and earth-fixed motion is written as

η=Rψv

with

Rψ=cosψ-sinψ0sinψcosψ0001

The vessel dynamics are formulated in the standard 3-DOF manoeuvring form

Mv+Cvv+Dvv=τH+τP+τR

where M is the effective inertia matrix, Cv contains Coriolis and centripetal terms, D(v) represents hydrodynamic damping, and τH, τP, and τR are the hull, propeller, and rudder contributions, respectively. In the present implementation, the inertia matrix is approximated as diagonal:

M=diagm11,m22,Iz

where m11 and m22 are the effective surge and sway inertias, and Iz is the effective yaw inertia. This form provides sufficient accuracy for horizontal-plane manoeuvring while preserving numerical robustness in long simulation rollouts.

The hull contribution is modelled through a combination of linear and nonlinear damping. Physically, these terms represent the resistance of the hull to forward motion, lateral drift, and yaw rotation. Linear damping improves low-speed transient matching, while nonlinear damping prevents unrealistic growth of velocity and yaw rate at higher operating conditions. In particular, additional linear surge damping is used to shape the early acceleration response, and linear yaw damping is used to regularise turning behaviour.

The propeller force is modelled using a speed-shaped thrust law rather than a constant-gain rpm2 formulation. A constant thrust model was found to be insufficient because it cannot simultaneously reproduce the measured early acceleration and the final speed plateau. The surge thrust is therefore expressed as

XPn,u=KTn|n|1+kbexp-uUb1+kdu2

where n is the commanded propeller speed, KT is the base thrust coefficient, kb is a low-speed thrust-boost coefficient, Ub is the characteristic speed over which the boost decays, and kd controls the reduction in effective thrust at higher surge speed. The exponential term increases thrust when the vessel is nearly stationary, thereby improving the launch behaviour, while the denominator prevents the model from over-predicting the final forward speed.

The rudder is represented as dynamic actuator with angle saturation and rate limitation. Let δc denote the commanded rudder angle and δ the actual rudder angle. The actuator dynamics are modelled as

δ=satδmaxδc-δ,      δ≤δmax

where δmax is the maximum rudder rate and δmax is the maximum rudder deflection. This prevents unrealistically abrupt turning and ensures that the hydrodynamic forces are generated by the actual rudder angle. The rudder force is driven by the effective inflow velocity at the rudder, denoted UR, and by the effective rudder angle of attack, denoted αR. A simplified normal-force model is used,

FN=kL12ρARUR2sinαR

where ρ is water density, AR is the rudder reference area, and kL is an empirical lift-scaling coefficient. The corresponding rudder contributions are separated into axial drag, lateral force, and yaw moment as

XR=-kXFN sinδ

YR=-kYFNcosδ

NR=-kNlRFNcosδ

where kX, kY, and kN are empirical scaling terms and lR is the effective rudder moment arm. This decomposition is important because it allows the model to control three physically different effects independently: loss of forward speed in a turn, lateral turning force, and yaw authority. In practice, this separation was necessary to match the observed turning-circle radius and the speed retained during turning without forcing all rudder-related behaviour to be controlled by a single parameter.

The total body-frame forces and moment are then written as

X=XH+XP+XR

Y=YH+YR

N=NH+NR

where XH, YH, and NH are the hull contributions. The corresponding equations of motion are solved numerically in time to update the vessel state.

u=fuu,v,r,X,       v=fvu,v,r,Y,      r=NIz

In the implementation, the full state vector is integrated using a fixed-step 4th order Runge-Kutta method.

x=u,v,r,ψ,δ,x,y⊤

This provides significantly better numerical stability than simple first-order integration, particularly when the model is used repeatedly inside the RL rollouts.

The control vector applied to the model is

a=nc,δc⊤

where nc is the commanded propeller speed and δc is the commanded rudder angle. Thus, the RL agent directly controls both the propulsion level and the steering input.

The parameter values used in the present experiments are summarised in Table X. These values correspond to the calibrated model employed throughout the simulation study.

| Parameter | Value |
| --- | --- |
| Maxx | 64.55 |
| Thrust coefficient | 0.06 |
| Drag coefficient | 1.5 |
| Low-speed thrust boost | 1.6 |
| High-speed thrust decay | 0.26 |
| Linear surge damp | 2.0 |
| Turn coefficient | 3.0 |
| Rudder force scale | 0.32 |
| Rudder yaw scale | 2.6 |
| Rudder X drag scale | 0.02 |
| Linear yaw damp | 1.5 |

The final hydrodynamic parameters are calibrated against experimentally measured straight-line and turning-circle manoeuvres of the Bluefin vessel. Straight-line trials were used to identify the surge-related parameters by matching forward-speed buildup and distance travelled, whereas turning-circle trials were used to identify yaw and rudder related parameters by matching turn timing, turning radius, and speed retention during turning. This calibration strategy ensures that the model is not only qualitatively plausible, but also quantitatively consistent with the measured manoeuvring characteristics of the experimental platform.

4-DOF Ship Model

The autonomous surface vessel is represented by a physics-informed manoeuvring model with four degrees of freedom (DOF): surge, sway, roll, and yaw. This formulation was selected because it preserves the dominant horizontal-plane manoeuvring behaviour required for path following and obstacle avoidance, while also capturing the coupling between lateral motion, roll response, and turning dynamics. Such coupling is important for small surface craft operating at low speed, where steering actions can generate measurable heel and lateral drift that influence the vessel trajectory.

Two coordinate systems are employed. The vessel pose is expressed in an earth-fixed frame as

η=x, y, ϕ, ψ⊤

where x and y denote the planar position, ϕ is the roll angle, and ψ is the heading angle. The body-fixed velocity vector is written as

ν=u, v, p, r⊤

where u is surge velocity, v is sway velocity, p is roll rate, and r is yaw rate. The complete vessel state is defined as

xs=u, v, p, r, x, y, ϕ, ψ, δ, n1, n2⊤

where δ is the actual rudder angle, n1 is the main propeller rotational speed, and n2 is the bow-thruster rotational speed. The control input vector is

uc=δc, n1c, n2c⊤

where δc is the commanded rudder angle, n1c is the commanded main propeller speed, and n2c is the commanded bow-thruster speed.

The Earth-fixed kinematics are expressed as

x=ucosψ-vsinψcosϕ

y=usinψ+vcosψcosϕ

ϕ=p

ψ=rcosϕ

Equations (5)-(8) transform the body-fixed velocities into Earth-fixed motion while accounting for the influence of roll on the projected sway and yaw motion.

The dynamic model is written by balancing the total hull, propeller, rudder, and bow-thruster contributions. The effective inertial terms are defined as

m11=m+mx,      m22=m+my,         m33=Ix+Jx,          m44=Iz+Jz

where m is the rigid-body mass, mx and my are added-mass terms in surge and sway, Ix and Iz are the rigid-body roll and yaw inertias, and Jx and Jz denote the corresponding added inertias.

The total non-dimensional forces and moments are decomposed as

Xd=XdH+XdP+XdR

Yd=YdH+YdR+YdB

Kd=KdH+KdH2+KdR+KdB

Nd=NdH+NdR-xGYd+NdB

where the subscripts H, P, R, and B denote the hull, propeller, rudder, and bow-thruster components, respectively. The rigid-body equations of motion are then written as

u=Xd12ρLdU2+m22vrm11

v=Yd12ρLdU2+m11urm22

p=Kd12ρLd2dU2+zH-zGmyv+mxurm33

r=Nd12ρL2dU2m44

U=u2+v2

where ρ is water density, L is vessel length, d is draft, xG and zG are the centre-of-gravity offsets, zH is the centre-of-pressure offset in roll, and U is the resultant body-frame speed.

The hull contribution is represented by nonlinear polynomial terms in drift angle, yaw rate, and roll angle. In physical terms, these terms describe the resistance of the hull to forward motion, lateral drift, turning, and heel. The roll dynamics also include hydrostatic restoring and roll damping terms, so that roll does not grow unrealistically during aggressive manoeuvres. The hydrostatic restoring moment is modelled through the metacentric height GM, producing the restoring coefficient

C44=g m GM

where g is the gravitational acceleration.

The main propeller force is modelled using an advance-ratio formulation. The advance ratio is defined as

J=1-wun1DP

where w is the wake fraction, DP is the propeller diameter, and n1 is the main propeller rotational speed. The propeller thrust coefficient is approximated by a quadratic polynomial,

KT=a0+a1J+a2J2

and the resulting surge-force contribution is proportional to

XdP∝1-tρn1n1DP4KT

where t is the thrust-deduction factor. This structure allows the propeller thrust to vary naturally with both shaft speed and vessel forward speed.

The rudder force is generated from the local inflow velocity and effective angle of attack. The rudder inflow is described by the components udR and vdR, leading to the resultant rudder inflow speed

UdR=udR2+vdR2

The effective rudder angle of attack is

αR=δ-tan-1-vdRudR

and the rudder normal force is approximated as

FdN=-ARLd6.13λ2.25+λUdR2sinαR

where AR is the rudder reference area and λ is the rudder aspect ratio. The rudder contributions are then resolved into surge, sway, roll, and yaw components as

XdR=1-tRFdNsinδcosϕ

YdR=1+aHFdNcosδcosϕ

KdR=zrYdRL

NdR=xR+aHxHFdNcosδcosϕ

where tR is the rudder thrust-deduction factor, aH is the hull-rudder interaction coefficient, and xR, xH, and zR are geometric rudder offsets. This decomposition allows the rudder to influence forward speed, lateral force, roll moment, and yaw moment in a physically meaningful way.

The bow thruster is modelled as a lateral force acting at an offset from the vessel centre. Its contribution is represented as

FBT∝ρn2n2KBT

where n2 is the bow-thruster rotational speed and KBT is the bow-thruster gain. The corresponding contributions to sway, roll, and yaw are written as

YdB=FBT,   KdB=zBFBT/L,   NdB=xBFBT/L

where xB and zB are the bow-thruster moment arms.

The actuator states are modelled dynamically rather than instantaneously. The rudder actuator is written as

δ=satδmaxδc-δ,     δ≤δmax

and the shaft-speed dynamics are presented as

n1=satnmaxn1c-n1,     n2=satnmaxn2c-n2

where δmax and nmax impose realistic actuator-rate limits. This is important because the RL policy acts on commanded controls, while the vessel responds according to the actual actuator states.

The complete state vector in Eq. (1) is integrated numerically using a fixed-step 4th order Runge-Kutta scheme (RK4). Internally, the simulation uses SI units and radians, while the RL environment maps normalised policy outputs to commanded rudder and shaft-speed values. The final hydrodynamic parameters are calibrated against experimentally measured straight-line and turning-circle trials of the Bluefin vessel by comparing simulated and measured motion metrics, including forward-speed buildup, distance travelled, turn timing, turning radius, and speed retention during turning.