Big picture: what the whole script is doing

The script is an automatic calibration tool for your simplified Python ship model.

It does this pipeline:

- load the real-vessel benchmark JSON files

- read the real control inputs S1, S2

- convert those real inputs into the simplified simulator’s inputs

  - rudder percent

  - rpm

- run the simulated ship model with many candidate parameter sets

- extract the same metrics from the simulated run

- compare sim metrics vs real metrics

- rank the parameter sets by error

So conceptually, it is:

real logs→replay in sim→measure→score→best parameters

1) Configuration section

This is the block near the top:

ROOT = Path(__file__).resolve().parent
REAL_TEST3_JSON = ROOT / "test_3_metrics.json"
REAL_TEST4_JSON = ROOT / "test_4_metrics.json"
OUTPUT_DIR = ROOT / "sweep_results"
SHIP_MODEL_MODULE = "ship_model"

What it does

It defines:

- where the input JSON files are

- where the output ranking files will be saved

- which Python file contains the ship model

Logic

You want the script to be reusable, so instead of hardcoding everything deep in the code, you keep file paths together at the top.

Then:

FIXED_MASS = 64.55

What it does

Keeps mass fixed while sweeping only:

- THRUST_COEF

- DRAG_COEF

- TURN_COEF

Logic

You told me you want to keep mass fixed for now, so this script only searches the other three.

Then:

THRUST_GRID = [...]
DRAG_GRID = [...]
TURN_GRID = [...]

What it does

These are the candidate values the sweep will try.

Logic

A parameter sweep means:

- choose a list of possible values for each parameter

- test all combinations

So if you have:

- 5 thrust values

- 5 drag values

- 6 turn values

then total runs:

5×5×6=150

That is why later the script uses itertools.product(...).

Then:

RPM_MAX = 30.0
RUDDER_CMD_MAX_PERCENT = 100.0

What it does

Defines the simplified simulator’s command range.

Logic

Your real logs use PWM-like S1 and S2, but the simplified ship model expects:

- rudder command in percent

- rpm value

So you need a mapping from real control signals to sim control signals.

Then:

OVERRIDE_S1_NEUTRAL = None
OVERRIDE_S2_NEUTRAL = None
OVERRIDE_S1_SCALE = None
OVERRIDE_S2_FULL_FWD = None

What it does

Allows you to manually override automatically inferred input mappings.

Logic

Sometimes the log-derived neutral values are messy. If you already know:

- true rudder neutral PWM

- true throttle neutral PWM

- true full-forward PWM

then you can manually set them here.

If left as None, the script tries to infer them from the real JSON.

Then:

WEIGHTS = {...}

What it does

Defines how important each error term is in the final score.

Logic

Not all metrics matter equally.

For example:

- if turning behavior is very important, give turn timing and yaw rate larger weight

- if speed peak is less important than transient acceleration, weight acceleration more

So the final score is a weighted sum of relative errors.

2) Utility functions section

These are small reusable math/helper functions.

wrap_180(deg)

def wrap_180(deg: float) -> float:
    return (deg + 180.0) % 360.0 - 180.0

What it does

Normalizes any angle difference into:

[-180∘,180∘)

Why needed

Angles wrap around.

Example:

- heading goes from 179° to -179°

- raw subtraction gives -358°

- physically the change is only 2°

So this function fixes that.

unwrap_heading_deg(yaw_deg)

What it does

Turns wrapped heading samples into a continuous heading trajectory.

Example:

- 170, 175, 179, -179, -175

becomes something like:

- 170, 175, 179, 181, 185

Why needed

To compute:

- total heading change

- time to 90°

- time to 180°

If you do not unwrap first, heading milestones become wrong.

sample_at_time(t_rel, values, query_s)

What it does

Returns the value of a signal at a chosen time using interpolation.

Example:

- what is u_body at 10 s?

- what is yaw rate at 5 s?

Why needed

Your time samples are not guaranteed to land exactly at 2.0, 5.0, 10.0 seconds, so you interpolate between points.

first_crossing_time(...)

What it does

Finds the first time a signal crosses a threshold.

Example:

- first time speed reaches 50% of peak

- first time speed reaches 90% of peak

Why needed

This gives rise-time metrics like:

- time_to_50pct_peak

- time_to_90pct_peak

first_abs_crossing_time(...)

What it does

Same idea, but uses absolute value.

Why needed

For turning, you usually care about magnitude of heading change:

- first time |heading change| reaches 90°

- first time |heading change| reaches 180°

because port and starboard should both work.

slope_over_window(...)

What it does

Fits a straight line over a time window and returns the slope.

Why needed

This is how the script estimates acceleration:

a≈dudt

over windows like:

- 0–2 s

- 0–5 s

Instead of using one noisy difference, it uses a fitted slope over several points.

cumulative_distance(x, y)

What it does

Computes the accumulated path length traveled.

Why needed

Distance is often a robust benchmark.

Instead of only looking at instantaneous speed, you can ask:

- how far has the vessel traveled after 10 s?

That is often more stable than raw speed samples.

circle_fit_radius(x, y)

What it does

Fits a circle to a trajectory arc and returns the radius.

Why needed

For turning calibration, one of the key metrics is:

- turning radius

- turning diameter

This function estimates that from the simulated path.

first_sustained_index(...)

What it does

Detects the first time a signal stays above a threshold for several consecutive samples.

Why needed

Without this, one noisy spike could falsely look like “motion start”.

This makes motion detection more robust.

Used for:

- detecting motion start from u_body

first_sustained_abs_index(...)

What it does

Same idea, but using absolute value.

Why needed

Used to detect:

- turn start from |yaw_rate|

safe_rel_error(sim, real, floor=1e-6)

What it does

Computes relative error safely.

error=∣sim-real∣∣real∣

with protection against divide-by-zero.

Why needed

The sweep needs one comparable error scale for different metrics.

Example:

- 0.2 m/s speed error

- 2 s time error

- 1 m radius error

Using relative error makes them comparable.

3) Data container classes

ReplaySeries

@dataclass
class ReplaySeries:
    t_sec: np.ndarray
    s1: np.ndarray
    s2: np.ndarray
    yaw_rate_real: np.ndarray
    u_body_real: np.ndarray

What it does

Stores the time series extracted from the real JSON.

Why needed

A dataclass makes the replay data easy to pass around cleanly.

Instead of passing 5 separate arrays everywhere, you pass one object.

ReplayMapping

This is very important.

It stores how to convert:

- real S1 → sim rudder percent

- real S2 → sim rpm

s1_to_rudder_percent()

Maps PWM-like rudder input to normalized rudder command.

s2_to_rpm()

Maps throttle PWM to rpm.

Why needed

Your real system and simplified sim use different control units.

The real logs contain:

- PWM-like values around 1000–2000

The simplified sim wants:

- rudder percent

- rpm

So this class is the bridge between real logged control and simulated control.

4) Loading helpers

load_json(path)

What it does

Reads a JSON file into Python dictionary form.

Why needed

Simple file-loading helper.

build_replay_series(data)

What it does

Takes one JSON dictionary and extracts the time-series arrays into a ReplaySeries.

Logic

The real JSON contains lots of summary info and series info.
This function picks out the raw arrays you need for replay:

- time

- s1

- s2

- real yaw rate

- real forward speed

infer_mapping(data, series)

This is one of the most important functions.

What it does

Automatically guesses:

- rudder neutral value

- throttle neutral value

- rudder scaling

- full-forward throttle value

Logic

Because the real PWM values are not directly meaningful to the simplified model, you need to infer:

s1_neutral

What PWM corresponds to “rudder centered”?

s2_neutral

What PWM corresponds to “no thrust”?

s1_scale

How much PWM change corresponds to full rudder range?

s2_full_fwd

What PWM corresponds to full-forward thrust?

If you do not set these manually, this function makes a best guess from the real data.

This is necessary so the sweep can replay the real controls into the simplified simulator.

5) Ship model loading and simulation

load_ship_model_module()

What it does

Imports ship_model.py dynamically.

Why needed

The sweep wants to modify the ship model’s parameters programmatically:

- thrust coefficient

- drag coefficient

- turn coefficient

- mass

So it loads the module, patches the global variables, and runs it.

simulate_replay(...)

This is the heart of the script.

What it does

For one candidate parameter set, it:

- loads the ship model

- writes the candidate parameters into that model

- creates a fresh ShipModel()

- replays the real S1/S2 series through the simplified simulator

- records simulated:

  - x

  - y

  - heading

  - yaw rate

  - forward speed

Logic step by step

a) Patch the model parameters

ship_model.MASS = ...
ship_model.THRUST_COEF = ...
ship_model.DRAG_COEF = ...
ship_model.TURN_COEF = ...

This makes the sim run with the current candidate set.

b) Recompute inertia

ship_model.MOMINERTIA = 0.5 * MASS * RUDDEROFFSET ** 2

Because in your simplified model yaw inertia depends on mass.

c) Create a fresh model

model = ship_model.ShipModel()

So each run starts clean.

d) Loop through the real log time series

For each timestep:

- compute dt

- convert S1 to rudder percent

- convert S2 to rpm

- call model.update(...)

This means the sim sees approximately the same control history as the real vessel.

e) Save the simulated outputs

It records:

- position

- heading

- yaw rate

- u_body

These are the signals later used for metric extraction.

6) Extracting metrics from the simulated run

extract_sim_metrics(sim)

What it does

Takes the simulated time series and computes the same kinds of metrics you already computed for the real logs.

It creates two groups:

- straight_metrics

- turn_metrics

Why needed

To compare real and simulated runs, they must be summarized in the same way.

Otherwise you would be comparing apples to oranges.

Straight metrics logic

detect motion start

Uses first_sustained_index(u, threshold=0.05, count=3)

This finds when the simulated vessel actually starts moving.

build motion-relative time

Then it redefines time so:

- t = 0 means motion start

compute:

- peak forward speed

- early acceleration

- distance after 10 s

- time to 50% peak

- time to 90% peak

- forward speed at 10 s

These are your surge benchmarks.

Turn metrics logic

detect turn start

Uses first_sustained_abs_index(yaw_rate, threshold=1.0, count=3)

So:

- t = 0 means actual turning onset

unwrap heading

Needed to compute heading milestones properly.

compute:

- peak absolute yaw rate

- time to 90°

- time to 180°

- turning radius from first 90°

- turning radius from first 180°

- forward speed at 10 s into turn

These are your turning benchmarks.

7) Scoring

score_against_real(...)

This is the function that converts “many metric differences” into one score.

What it does

It compares simulated metrics to the real benchmarks and computes relative errors for:

Speed

- peak speed

- accel 0–2 s

- accel 0–5 s

- distance at 10 s

- t50

- t90

Turn

- t90

- t180

- peak yaw rate

- radius first 90°

- radius first 180°

- speed at 10 s into turn

Then it multiplies each by its weight and sums them.

Logic

Without one scalar score, you cannot rank parameter sets automatically.

So this function is the “objective function” of the calibration.

Lower score = closer match.

8) Main sweep loop

main()

This is the top-level controller.

What it does

a) Create output folder

So results can be saved.

b) Load real benchmark JSON files

For test 3 and test 4.

c) Build replay series

Extract raw control and response traces.

d) Infer control mappings

Figure out how real PWM maps to sim input units.

e) Build all parameter combinations

Using:

itertools.product(THRUST_GRID, DRAG_GRID, TURN_GRID)

This produces every possible combination.

f) For each combination

- simulate test 3 replay

- simulate test 4 replay

- extract sim metrics

- compare against real metrics

- save score + details

g) Sort by score

So best candidates appear first.

h) Save results

As:

- ranked CSV

- top 10 JSON

9) Why two separate replays are used

Notice this part:

sim3 = simulate_replay(series3, ...)
sim4 = simulate_replay(series4, ...)

Why

Because:

- test 3 is the speed test

- test 4 is the turning test

Then later:

fused_metrics = {
    "straight_metrics": sim3_metrics["straight_metrics"],
    "turn_metrics": sim4_metrics["turn_metrics"],
}

Logic

You want:

- speed fit from the speed test

- turn fit from the turning test

That is cleaner than trying to judge everything from one test.

10) Why this method is useful

The logic behind the whole script is:

Instead of saying:

- “I think thrust should be 0.06”

- “Maybe turn coefficient should be 800”

you let the script systematically test many combinations and rank them objectively.

This is a very standard calibration approach:

- define candidate parameters

- replay real experiment

- extract same metrics

- compute error

- choose best set

11) What assumptions this script makes

This is important.

The script assumes:

1. The simplified ship model is still valid enough

It assumes that by tuning 3 parameters, the simplified model can approximately reproduce the real vessel.

2. Control mapping is reasonable

It assumes your guessed mapping from S1/S2 to rudder/rpm is close enough.

3. The chosen metrics are representative

It assumes:

- speed rise

- speed peak

- yaw timing

- turn radius

are the right things to match.

12) What you would change in the future

If you reproduce this script later, the usual things you might change are:

Change the parameter grids

If you find the best thrust is around 0.06, next time use:

- 0.055, 0.058, 0.060, 0.062, 0.065

instead of a coarse grid.

Change weights

If turning matters more than straight speed, increase turn weights.

Improve control mapping

If you know exact PWM-to-rpm calibration, replace the guessed mapping.

Add more parameters

You could later sweep:

- MASS

- rudder offset

- separate yaw inertia

- rudder rate limit

Upgrade the metric extractor

You could compare full time histories, not only summary metrics.

13) Short summary of each function

Here is the compact memory version:

- wrap_180 → fix angle differences

- unwrap_heading_deg → make heading continuous

- sample_at_time → interpolate signal at chosen time

- first_crossing_time → when signal first reaches threshold

- first_abs_crossing_time → same for absolute value

- slope_over_window → estimate acceleration/slope

- cumulative_distance → total traveled distance

- circle_fit_radius → estimate turning radius

- first_sustained_index → detect motion start robustly

- first_sustained_abs_index → detect turn start robustly

- safe_rel_error → compare sim vs real safely

- ReplaySeries → container for real replay signals

- ReplayMapping → convert real control to sim control

- load_json → read JSON

- build_replay_series → extract time-series arrays

- infer_mapping → guess neutral/full-range control mapping

- load_ship_model_module → import ship model dynamically

- simulate_replay → run sim using real control series

- extract_sim_metrics → measure sim the same way as real logs

- score_against_real → convert metric mismatches to one score

- main → run the whole sweep and save results

14) The most important logic to remember for future reproduction

If you forget the code, remember this structure:

Calibration recipe

- Collect real test logs

- Extract control and response signals

- Replay the real controls in simulation

- Extract the same metrics from simulation

- Compute mismatch

- Sweep parameters

- Choose best set

That is the reusable idea.