Bluefin Log Parser
Code Breakdown and Integration Notes

Document generated: 2026-01-29
Covered file: log_parser.py

# Environment overview

This document explains how the Bluefin log parser works, step-by-step, including (1) the log format, (2) how the parser converts log lines into structured frames, and (3) how those frames can be mapped into the observation format expected by your RL Gymnasium environment.

The parser supports four line types in the log:

HDG line: a heading reference (used only for checking SLAM accuracy).

Pose line: SLAM output (X, Y, yaw).

LiDAR line: 360-degree scan of 720 ranges (0.5 degree per beam).

RC line (optional): manual controller inputs S1 (rudder) and S2 (thruster).

A ‘frame’ is emitted when a LiDAR line arrives. The frame combines the most recent pose (and derived velocity) with the LiDAR scan, plus any last-seen HDG/RC values.

# 1. Install and import dependencies

The log parser is designed to be lightweight. It relies on Python standard libraries plus NumPy for efficient array handling.

import re import numpy as np from dataclasses import dataclass from typing import Optional, Tuple, Dict, Any

Why these packages are used:

re: pattern matching to recognise which log line type we are reading (HDG / Pose / LiDAR / RC).

numpy: efficient conversion of the LiDAR list into an array, fast rolling (rotation), clipping, and velocity magnitude.

dataclasses: convenient container (BluefinFrame) for one timestep.

typing: makes the code self-documenting (Optional, Tuple, Dict, return annotations).

# 2. Understanding type hints and the ‘->’ symbol

In Python, the syntax ‘-> something’ on a function definition is a return annotation. It does not change runtime behaviour by itself; it is mainly for readability and tooling (IDE auto-complete, static type checkers like mypy).

def _ts_to_seconds(ts_str: str) -> float: ...

In this example:

ts_str: str means the function expects ts_str to be a string.

-> float means the function is expected to return a floating-point number.

If you wrote def _ts_to_seconds(ts_str): it would still run, but you lose the ‘self-documenting’ benefit.

Optional[T] means “either a value of type T, or None”. This is commonly used for state variables that start empty and later get filled (latched).

from typing import Optional last_pose: Optional[Tuple[float, float, float]] = None

# 3. Log format recap (with examples)

Typical log lines look like:

[13:32:07.817313][01935] HDG:350.75 [13:32:07.824416]-000.0,+000.0,+014.4 [13:32:07.915416][ 0, 0, 0, ... ] [13:32:08.220195][02338] S1:1542 S2:1000 RC

Important details:

Timestamps are HH:MM:SS.microseconds and update at ~10 Hz.

Pose values are in metres; yaw is in degrees.

LiDAR values are in decimetres (dm). The parser converts dm to metres by multiplying 0.1.

LiDAR range is 1–16 m; out-of-range is logged as 0 (optionally replaced with max range in the parser).

# 4. Data container: BluefinFrame

BluefinFrame is a dataclass that stores all information for one timestep (one LiDAR scan). Using a dataclass keeps the code clean: you define fields once, and Python auto-generates __init__, __repr__, etc.

@dataclass class BluefinFrame: t_sec: float ts_str: str x_m: float y_m: float yaw_deg: float vx_mps: float vy_mps: float speed_mps: float lidar_m: np.ndarray hdg_ref_deg: Optional[float] = None s1: Optional[int] = None s2: Optional[int] = None seq: Optional[int] = None

Field explanation:

t_sec: Time (seconds) since the start of the run. Computed from the log timestamp.

ts_str: Original timestamp string from the log, kept for debugging.

x_m, y_m: Position in the world/map frame (metres).

yaw_deg: Heading/yaw in degrees, normalised to [0, 360).

vx_mps, vy_mps: Velocity in the world frame (m/s), derived from consecutive pose samples.

speed_mps: Speed magnitude U = sqrt(vx^2 + vy^2).

lidar_m: LiDAR ranges in metres, shape = (N,) where N depends on downsampling.

hdg_ref_deg: Optional heading reference from the HDG line (used for checking SLAM, not control).

s1, s2: Optional manual control signals (logged for reference).

seq: Optional sequence number from the HDG/RC lines.

# 5. Helper functions

Helper functions keep the decoder readable. Each helper does one small job: time conversion, angle wrapping, parsing LiDAR arrays, rotation, and downsampling.

## 5.1 _ts_to_seconds(ts_str: str) -> float

def _ts_to_seconds(ts_str: str) -> float: hh, mm, rest = ts_str.split(":") ss, micros = rest.split(".") return int(hh)*3600 + int(mm)*60 + int(ss) + int(micros) / 1e6

Purpose: convert HH:MM:SS.microseconds into a single floating-point value in seconds.

Split the string by ':' into hours, minutes, and the 'seconds.microseconds' part.

Split seconds.microseconds by '.' into integer seconds and integer microseconds.

Compute total seconds = hours*3600 + minutes*60 + seconds + microseconds/1e6.

Using float seconds makes it easy to compute dt for velocity.

## 5.2 _wrap_360(deg: float) -> float

def _wrap_360(deg: float) -> float: return (deg % 360 + 360) % 360

Purpose: normalise any angle (degrees) into the range [0, 360).

deg % 360 already wraps, but can be negative if deg is negative depending on language conventions.

The extra '+ 360' then '% 360' ensures the final output is always within [0, 360).

Example: -10 degrees becomes 350 degrees.

## 5.3 _parse_int_list_csv(text: str) -> np.ndarray

def _parse_int_list_csv(text: str) -> np.ndarray: parts = text.split(",") out = np.fromiter((int(p) for p in (x.strip() for x in parts) if p != ""), dtype=np.int32) return out

Purpose: convert the comma-separated LiDAR list into a NumPy integer array.

text.split(',') breaks the long string into tokens.

x.strip() removes spaces around each token.

int(p) converts each token into an integer.

np.fromiter builds an array efficiently without building an intermediate Python list.

## 5.4 _rotate_lidar_by_degrees(lidar_m, degrees) -> np.ndarray

def _rotate_lidar_by_degrees(lidar_m: np.ndarray, degrees: float) -> np.ndarray: if degrees == 0: return lidar_m shift = int(round(degrees * 2)) # 0.5 deg per beam => 2 beams per degree return np.roll(lidar_m, shift)

Purpose: rotate a 360-degree LiDAR scan if your “beam 0” convention differs between the real sensor and the simulation.

Your LiDAR has 720 beams over 360 degrees: each beam is 0.5 degrees.

Rotating by 1 degree means shifting by 2 array elements.

np.roll performs a circular shift: values that roll off the end wrap around to the beginning.

Use lidar_angle_offset_deg in the decoder constructor to configure this without changing code.

## 5.5 _downsample_stride(arr, out_n) -> np.ndarray

def _downsample_stride(arr: np.ndarray, out_n: int) -> np.ndarray: n = len(arr) if out_n == n: return arr if out_n <= 0: raise ValueError("out_n must be > 0") step = n / out_n idx = (np.arange(out_n) * step).astype(int) return arr[idx]

Purpose: reduce a long LiDAR array (e.g., 720 beams) to fewer beams (e.g., 63 beams) by sampling evenly.

If out_n equals the input length, return the original array (no work).

Compute step = n/out_n (e.g., 720/63 ≈ 11.43).

Create indices [0*step, 1*step, 2*step, ...] and cast to int.

Index the original array with those indices.

This preserves the full 360-degree coverage, but at lower angular resolution.

Important: your current file had a typo np.arrange(...) (should be np.arange(...)). If you plan to downsample, make sure this is corrected.

# 6. Streaming decoder: BluefinStreamDecoder

BluefinStreamDecoder is designed for both offline parsing (reading a log file) and online parsing (reading live UDP messages). You feed it one line at a time. When it sees a LiDAR line, it returns a complete BluefinFrame object; otherwise it returns None.

## 6.1 Regex patterns (line recognition)

The decoder uses four compiled regular expressions (regex) to recognise each line type.

_re_hdg = re.compile(r"^\[(?P<ts>\d{2}:\d{2}:\d{2}\.\d{6})\]\[(?P<seq>\d+)\]\s*HDG:(?P<hdg>[-+]?\d+(?:\.\d+)?)\s*$") _re_pose = re.compile(r"^\[(?P<ts>\d{2}:\d{2}:\d{2}\.\d{6})\](?P<x>[-+]\d+\.\d+),(?P<y>[-+]\d+\.\d+),(?P<yaw>[-+]\d+\.\d+)\s*$") _re_rc = re.compile(r"^\[(?P<ts>\d{2}:\d{2}:\d{2}\.\d{6})\]\[(?P<seq>\d+)\]\s*S1:(?P<s1>\d+)\s*S2:(?P<s2>\d+)\s*RC\s*$") _re_lidar = re.compile(r"^\[(?P<ts>\d{2}:\d{2}:\d{2}\.\d{6})\]\[(?P<body>.*)\]\s*$")

Key regex symbols used in these patterns:

| Symbol | Meaning |
| --- | --- |
| ^ | Start of line |
| $ | End of line |
| \[ and \] | Match literal '[' and ']' characters |
| \d | A digit (0–9) |
| {2} | Exactly 2 repeats (e.g., \d{2} matches two digits) |
| + | One or more repeats |
| * | Zero or more repeats |
| ? | Optional (0 or 1 repeats) |
| . | Any character except newline |
| \s* | Zero or more whitespace characters |
| ( ... ) | A capturing group |
| (?P<name> ... ) | A named capturing group (accessed as m.group('name')) |
| (?: ... ) | A non-capturing group (grouping without saving as a numbered group) |

Named groups are the most important idea here. For example, the HDG pattern captures the timestamp as group 'ts', the sequence number as group 'seq', and the heading as group 'hdg'.

## 6.2 Time origin and ‘latched’ values

The decoder keeps a time origin (t0) and several latched values. “Latched” means the decoder stores the most recent value it has seen, so that when a LiDAR scan arrives it can package everything into one frame.

self._t0: Optional[float] = None self._last_hdg_ref: Optional[float] = None self._last_seq: Optional[int] = None self._last_s1: Optional[int] = None self._last_s2: Optional[int] = None self._last_pose: Optional[Tuple[float, float, float]] = None self._last_pose_t: Optional[float] = None self._last_vel: Tuple[float, float, float] = (0.0, 0.0, 0.0)

At the first timestamp, _t0 is set. Every later timestamp is converted to t_sec = ts_seconds - _t0.

_last_pose stores the most recent (x, y, yaw).

_last_pose_t stores the time of that pose, so dt can be computed next pose update.

_last_vel stores the most recently computed velocity (vx, vy, speed).

_last_hdg_ref / _last_s1 / _last_s2 / _last_seq store the last heading reference and last manual inputs.

Optional[...] indicates the variable starts as None (unknown) until the first matching line arrives.

## 6.3 _real_time(ts_str) -> float

def _real_time(self, ts_str: str) -> float: sec = _ts_to_seconds(ts_str) if self._t0 is None: self._t0 = sec return sec - self._t0

Purpose: convert a timestamp string into elapsed seconds since the beginning of the run.

Convert HH:MM:SS.microsec to absolute seconds (sec).

If this is the first timestamp, set _t0 = sec.

Return elapsed time = sec - _t0.

## 6.4 feed(line) -> Optional[BluefinFrame]

feed() is the core function. It takes one line at a time, updates internal state, and returns a frame only when a LiDAR line is received.

def feed(self, line: str) -> Optional[BluefinFrame]: line = line.strip() if not line: return None if line matches RC: latch S1, S2, seq return None if line matches HDG: latch hdg_ref, seq return None if line matches POSE: compute elapsed time t latch pose (x, y, yaw) update velocity from previous pose return None if line matches LIDAR: compute elapsed time t parse lidar list -> array convert units + clip + rotate + downsample if no pose yet: return None return BluefinFrame(t, pose, velocity, lidar, last_ref_values)

Step-by-step explanation:

Step 0: Clean the line: strip() removes leading/trailing whitespace; empty lines are ignored.

Step 1: Check RC lines first: RC lines are optional. If present, store (S1, S2) and the sequence number.

Step 2: Check HDG lines: Store heading reference (for debugging) and sequence number.

Step 3: Check Pose lines: Parse x, y, yaw. Convert timestamp to elapsed time. Compute velocity if a previous pose exists.

Step 4: Check LiDAR lines: Parse 720 integers, convert to metres, replace zeros if configured, then output a BluefinFrame.

Why is the frame emitted on the LiDAR line? In your dataset, the LiDAR scan is the largest payload and also defines the observation for the RL agent. Pose/HDG/RC lines are ‘supporting’ values that can be latched until the next scan.

# 7. Offline decoding: frames_from_file()

def frames_from_file(filepath: str, decoder: Optional[BluefinStreamDecoder] = None): if decoder is None: decoder = BluefinStreamDecoder() with open(filepath, "r", errors="ignore") as f: for line in f: frame = decoder.feed(line) if frame is not None: yield frame

This function is a generator: it yields frames one-by-one as they are decoded. This is memory-efficient for large logs because it does not load the whole file into RAM.

If no decoder is provided, create a default BluefinStreamDecoder().

Read the file line-by-line.

Call decoder.feed(line).

If a frame is produced (LiDAR line), yield it to the caller.

# 8. Convert a frame to an RL observation: frame_to_gym_obs()

def frame_to_gym_obs( frame: BluefinFrame, *, origin_xyh: Optional[Tuple[float, float, float]] = None, include_velocity: bool = True, ) -> Dict[str, Any]: ...

Purpose: map a decoded BluefinFrame into the dictionary observation format used by your Gymnasium environment.

Key features:

Origin shift: if origin_xyh is given, the first frame’s (x, y, yaw) can be treated as (0, 0, 0).

The LiDAR array is stored under the key 'lidar'.

Position is stored as a 2D vector 'pos' = [x, y].

Heading is stored as a 1D array 'hdg' = [yaw].

Optional: include velocity 'vel' and speed 'spd' in the observation.

Target-related fields are placeholders right now (set to zeros).

Integration note: your simulation environment (ASVLidarEnv) uses the key name 'dhdg' for heading rate. In your log_parser.py, the key is currently 'dhg'. To integrate cleanly, make sure these names match.

# 9. Running the script (offline)

From a terminal in the same folder as log_parser.py, run:

python log_parser.py test_1.log

What happens:

The script creates a decoder (default 720 beams).

It loops through frames_from_file(...).

It sets the first frame pose as the origin.

It prints decoded frames to the terminal.

# 10. Using the parser online (real-time UDP)

The same decoder can be used for live communication. The only difference is the source of lines: instead of reading from a file, you read from a UDP socket, split incoming bytes into lines, and feed each line.

import socket from log_parser import BluefinStreamDecoder, frame_to_gym_obs decoder = BluefinStreamDecoder(lidar_out_beams=63) # example: match simulation beams sock = socket.socket(socket.AF_INET, socket.SOCK_DGRAM) sock.bind(("0.0.0.0", 5000)) buffer = "" origin = None while True: data, addr = sock.recvfrom(65535) buffer += data.decode(errors="ignore") while "\n" in buffer: line, buffer = buffer.split("\n", 1) frame = decoder.feed(line) if frame is None: continue if origin is None: origin = (frame.x_m, frame.y_m, frame.yaw_deg) obs = frame_to_gym_obs(frame, origin_xyh=origin, include_velocity=True) print("obs keys:", obs.keys(), "pos:", obs["pos"])

This loop only observes the incoming data (no control commands sent). Once observation-only works reliably, the next step is to add an action-to-command encoder (send rudder/thruster values back to the vessel).

# 11. Common upgrades and checks

Downsampling: Set lidar_out_beams to match your RL observation size (e.g., 63).

Beam alignment: If beam 0 is not aligned, use lidar_angle_offset_deg to rotate the scan.

Out-of-range handling: Decide whether 0 should mean ‘max range’ or ‘unknown’; configure lidar_out_of_range.

Velocity smoothing: If SLAM is noisy, consider a low-pass filter on vx, vy.

Heading rate (dhdg): Compute dhdg from consecutive yaw angles and dt if your agent needs it.

Coordinate frames: If your controller expects body-frame velocity, rotate (vx, vy) by -yaw.

# References (official docs)

If you want authoritative references for the concepts used in this file, these are useful starting points:

Python regular expressions (re module): https://docs.python.org/3/library/re.html

Python socket programming (socket module): https://docs.python.org/3/library/socket.html

Python dataclasses: https://docs.python.org/3/library/dataclasses.html

Python typing module (Optional, Tuple, etc.): https://docs.python.org/3/library/typing.html

PEP 3107 – Function Annotations: https://peps.python.org/pep-3107/

PEP 484 – Type Hints: https://peps.python.org/pep-0484/

NumPy documentation: https://numpy.org/doc/